Output

Counter

Program for COUNTER :

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity counter is

Port ( clk : in  STD_LOGIC;

count : out  STD_LOGIC_VECTOR (3 downto 0));

end counter;

architecture Behavioral of counter is

signal s:std_logic_vector(3 downto 0):="0000";

signal s1:std_logic_vector(20 downto 0):=(others=>'0');

signal sclk:std_logic:='0';

begin

process(clk)

begin

if(clk'event and clk='1')then

s1<=s1+1;

end if;

end process;

sclk<=s1(20);

process(sclk)

begin

if(sclk'event and sclk='1')then

s<=s+1;

end if;

end process;

count<=s;

end Behavioral;

UCF for COUNTER :

NET "clk" LOC="p181";

NET "count<0>" LOC="p80";

NET "count<1>" LOC="p79";

NET "count<2>" LOC="p78";

NET "count<3>" LOC="p77";